AmigaControl 0.1

1996 - Arve Hjnnevg

-----------------------

AmigaControl is a small application that allows you to share your BeBox

keyboard and monitor with an Amiga computer. The application uses the

geekport to control a monitor switch and to send keycodes to the Amiga.

The settings file let you specify which pins you connect, but if you don't

change anything, use the following hardware:

Keyboard connection:

Geekport:

G VCC            CLK DG

|  |     +--+--+--|  |

8  7  6  5  4  3  2  1

23 22 21 20

|  |

MS  DATA

Amiga 4000 keyboard connector:

CLK

/

6   5

|

4     2 - DG

2   1

\

DATA

(Other models have a different connector, but uses the same protcol.

Check your documentation for pinouts)

NOTE: You may have to use resistors from CLK to DG and DATA to DG to

get the 0 output low enough.

Monitor switch:

Use MS to control switch. MS=5v => Amiga output is selected.

MS=0v or open => BeBox is output selected.

My setup:

2 5v relays.

1 hex inverter. (7404)

12-1   14 - VCC        MS - resistor (around 1k)

/2   13 - MS          G -/

\3   12 \-1

Relay2-4   11 /

5   10 \

6   9  /

G-7   8  - Relay1

Relay:

Monitor (R G B HS VS)

VCC   -\   |  ....

-------

Relayx-/   |\ ....

Amiga Bebox  (R G B HS VS)

Software:

Installation:

Edit AmigaControl_settings if needed and move it to:

/boot/system/settings

Setting file:

For the above hardware, leave the first fields alone.

However, you may want to change the keymap. Each entry maps a Be

keycode to an Amiga keycode or to a special function. The Be

keycode mapped is the same a the Entry's position in the map. A

positive entry in this keymap maps to an Amiga keycode. -1 is no

action, and -2 toggles the monitor switch (F12 in the supplied file).

The Amiga keyboard reset is currently hardcoded to lCtrl-lAlt-rAlt.

Future:

Make keyboard communiction a realtime thread.

Send questions and bug reports to <arve@cs.ucsb.edu>

Arve Hjnnevg

--------------------------------------------------------

775 Camino Del Sur Apt. C7   |  Nordre Knappen 91

Isla Vista, CA 93117         |  5355 Knarrevik

USA                          |  NORWAY

(valid until june97)         |

--------------------------------------------------------