This is the wiring diagram for the cable necessary to link two

machines together by their parallel ports.  This is the same wiring

diagram used by a Turbo LapLink cable, available for sale from many

PC parts outlets.

1 - 1

2 - 15

3 - 13

4 - 12

5 - 10

6 - 11

7 nc

8 nc

9 nc

10 - 5

11 - 6

12 - 4

13 - 3

14 - 14

15 - 2

16 - 16

17 - 17

18 nc

19 nc

20 nc

21 nc

22 nc

23 nc

24 nc

25 - 25 (ground)