F L E X Y   V 3.0

INTEGRATED

SIMULATION & ANIMATION

ENVIRONMENT FOR FMS

INSTRUCTIONS FOR USE

1. INTRODUCTION

FLEXY  represents an integrated environment for simulation and animation of flexible manufacturing systems (FMS). The software is envisaged for use on PC 386 or 486 computers with VGA/EGA graphics. The main components of FLEXY system are:

FMS simulator -  universal data driven simulation model enabling the simulation of different flexible manufacturing systems. Model of simulated system is defined only by input data (no programming required).

Simulation data entry module enabling input of data describing simulated system.

Graphic module providing following functions:

- creation of graphic symbols for machining stations

- simulated system layout design

- animation of simulated system

Reporting system with following functions:

- review/printing of input data

- review/printing of simulation results.

2. INSTALLATION

FLEXY system is distributed on one floppy disk. Complete system is packed in a file flexy.zip. For the installation of FLEXY it is necessary to do the following:

1. create appropriate directory (using DOS commands)

2. Copy files

Flexy.zip

pkunzip.exe

from distribution disk

3. enter the command for unpacking:

pkunzip flexy

3. USING THE SYSTEM

FLEXY system is activated by entering flexy command at DOS prompt. After starting the system the main menu appears on the screen. The following functions can be selected from the menu:

Creation of machine symbol

Layout drawing

Input of simulation data

Simulation

Animation

Review of input data

Review of simulation results

Required function is selected by clicking left mouse button on desired function or by pressing an emphasized letter in desired function.

3.1. CREATION OF MACHINE SYMBOL

This module enables user to create machine symbols that will be used for layout designing. Symbol of machine is created by combining geometric entities offered in appropriate menu. To draw an entity select it from the menu and put it at desired position in the drawing field (Fig. 1).

For the selection of entity and for drawing use left mouse button.

The following geometric entities are available:

LINE

RECT (rectangle)

FILARC (filled-in circle)

CIRCLE

BAR (filled-in rectangle)

The following drawing line types can be selected:

SOLID

DOTTED

CENTER

DASHED

Figure 1

drawing area

fill-in patterns

drawing

entities

active

colour

colour

pallete

Drawing colour is selected from palette at the right side of the screen.

Hatch pattern (for filled-in entities) is selected from the hatch menu. Before selecting the hatch pattern desired colour should be selected.

Deletion of entities is enabled by the ERASE function. After selection of this function each pressing of the left mouse will cause deletion of the last entity in the chain. To exit ERASE function press the right mouse button.

Figure 2

File managing is provided by FILE function (Fig. 2). Each machine symbol is stored in separate file under given name with mch extension (extensions are assigned by the system). FILE function enables opening new files, retrieving existing files, saving currently drawn files and exiting the module for machine symbol creation. Left mouse button is used for selection and right button acts as <ENTER>.

3.2. LAYOUT DRAWING

After selection of this function field for layout drawing and functional menu appear (Fig. 3). The functional menu includes the following functions:

MACHINE	- drawing of machines and other equipment,

LINE		- drawing of transportation paths,

NODE	- definition of transport network nodes,

SPRITE	- definition of movable figures (AGVs).

Figure 3

It is recommended to start layout drawing with definition of transportation paths using LINE function. The next step would be determination of nodes in transportation network (NODE function). Nodes are points where transportation path changes its direction and point in front of machining stations where transportation device should stop for interchange of workpieces.

MACHINE function is used for entering machine symbol at appropriate place in layout. When this function is selected the list of existing machine symbols appears on the screen. Appropriate symbol is selected the list by using mouse. The symbol is placed at the proper place in the layout by defining two diagonal vertices of rectangle in which the symbol should be drawn. After drawing in the machine the machine number (facility number) should be entered from the keyboard (prompt will appear in the lower right corner of the screen). The next three steps are:

Definition of the loading point - during the animation the workpiece entering machine will appear at this point.

Definition of the unloading point - finished part waiting for transportation will appear here.

Definition of machining point - the place at which the workpiece is machined.

In this way machine definition is completed. Above procedure is repudiate until all stations in the system are defined. To change current machine symbol it is necessary to select MACHINE function again otherwise not.

Note: Each layout should contain input station (place where parts are entering into the system), output station (place where parts are leaving the system) and buffer. The following facility numbers should be used for these stations:

INPUT STATION	- 103

OUTPUT STATION	- 106

BUFFER		- 107

These stations are created in the same manner as the machines are created.

SPRITE function is used for the definition of movable elements - transportation devices. When this function is activated first the list of existing machine symbols appears. The symbol for AGV should be chosen from the list. The selected symbol should be placed in the special field for AGV design. After that the total number of devices in the system should be entered. Then the starting position of AGVs - the nodes in which the vehicles stand at the beginning of the work are defined.

Note: SPRITE function is not used for the systems with conveyor transportation

Deletion of entities and file management is similar as for creation of machine symbols. For layout files extension lay is used.

3.3. SIMULATION DATA INPUT

This function is used for the entrance of numerical data describing simulated system. The following data should be entered:

The total number of machining stations

The number of the groups of machines - Group of machines is a set of machining stations with the same manufacturing task (machines of the same type performing the same operations). If within a system each machine performs different operations then the number of the groups is the same as the number of machining stations i.e. each machine represents a separate group.

The definition of machines constituting each group follows:

Number of machines in group

Machines constituting group - The numbers of machines belonging to group are successively entered (each station has a unique facility number).

After the definition of machines and groups of machines transportation system is described.

The number of transport vehicles - 0 is entered for conveyor transport. Maximum number of AGVs is 3.

The speed of transport vehicles

The number of preparation stations (for palletization and repalletization)

Definition of workpiece data follows.

The number of different workpiece types

For each part the following should be entered:

Batch size

Number of operations - under operation the machining of a workpiece on one machine is understood.

Preparation time - the time spent by a workpiece on preparation station before going to the machine. If the preparation time between two operations is given it is understood that the part is moved to preparation station for repalletization. If preparation time is zero the part goes directly from one machine to another.

Machining time - the duration of an operation (the time the workpiece spends on a certain machine)

Group of machines - a group of machines on which the operation is performed.

Priority - If predefined priorities are used the number defining the priority of the workpiece should be entered (greater number means higher priority). If scheduling rules SPT or FIFO would be used the zero should be entered here.

After entrance of workpiece data the following is to be defined:

Interarrival time of workpieces - care should be taken not to give to short interarrival time. In this way generation of too large number of transactions which may lead to a blockade (simulator may control the maximum of 400 transactions at a time) will be avoided.

Priority of parts processing - one of following priority rules may be chosen:

SPT 		parts which have passed a greater number of operations get the higher priority

FIFO		priority is assigned in accordance with the order of workpiece entrance in the system.

FIXED		defined by the user (in accordance with previously entered priorities for each workpiece)

The order of entrance of workpieces- workpieces may enter the system randomly or in turn.

The layout description then follows.

The number of nodes in the transport network - the number of nodes through which transport network is defined. The nodes represent the points in which the vehicles are stopping for exchange of parts and the points in which their movement direction is changed.

For each pair of nodes linked with transport path the distance should be determined in the following way:

Starting node - the number of the starting node (the nodes should be numbered in the same manner as during the layout drawing)

Ending node - the number of the ending node.

The distance - the distance between nodes in adopted units of length.

The order of entrance of nodes determines the direction of movement. For instance if it is entered

Starting node 		1

Ending node		2

Distance		9

This would mean that on the part of the path between the nodes 1 and 2 vehicles may move only in the direction from 1 towards 2. Should a part of a path be defined as bi-directional then the distance between nodes should be twice defined, with inverse sequence of nodes. For instance if a part of the path between the nodes 1 and 2 is bi-directional then the following should be entered

Starting node 		1

Ending node		2

Distance		9

and

Starting node 		2

Ending node		1

Distance		9

The end of entrance of distances between nodes is determined by entering the number 999 for the starting node.

Note: For conveyor transport use only one-way paths.

Warning: Transportation paths should be continuous (without blind ends or disjoints) otherwise the system will be halted during the simulation

Further the position of machining stations and vehicles should be defined. For each vehicle the following should be entered:

Starting node - the node in which the vehicle is positioned at the beginning of the simulation.

After that for each machining station the number of the node in which the vehicle will stop during the exchange of parts should be given. This achieved by entering:

The number of station (facility number)

and

The number of node in front of station

It should be borne in mind that it is important to define the points in which are situated:

INPUT STATION	- 103

OUTPUT STATION	- 106

BUFFER		- 107

Note: The compatibility of data entered during the layout drawing and those entered during the definition of the layout should be borne in mind otherwise the errors during animation should occur.

Now the data about machining stations are entered. For each machine the following has to be determined:

The capacity of machine's buffer - the greater number of parts that may be in the buffer of machine including the one that is machined.

The reliability of the machine - the probability of machine work without failure. On the basis of given reliability during the simulation some random number of failures is generated.The maximum reliability is 1.00, and the machine works without failures.

The number of failures - there exists possibility to enter fixed number of failures to be generated. If this option is used then 0 should be entered for the reliability of the machine. If 0 is entered both for machine's reliability and for the number of failures it is considered that the machine works without failures.

Machine repair time - when possibility of failure is envisaged mean repair time of the machine should be entered.

The robot servicing the machine - enter the number of the robot servicing the machine (numbering begins from 1). Zero is entered when there is no robot servicing machine that is parts are transferred directly from AGV (conveyor) to the machine.

Robot's speed

Manipulation time

The last data entered is

Maximum operation time of the system - determining the maximum duration of simulation

Note: During the entrance of data, in case of error, with arrows on the keyboard corrections may be made on the current screen. Pressing ESC will return previous screen/page.

Extension inp is assigned to input data file. Starting the input data function with the existing file name means that changes in existing file should be done.

3.4. SIMULATION

By choosing this function from the menu the simulation without animation is started. At the beginning the input data file should be selected. The simulation is activated. It is used for the generation of statistical reports for the simulated system (utilization of machines, queues, failures, etc.). During the simulation current state of the simulation clock and number of finished parts are displayed on the screen.

3.5. ANIMATION

First the simulation input file should be selected. Then simulator generating data for animation is activated. After that the layout file name should be selected. Finally the animationof simulated system starts (Fig 4).

3.6. REPORTS

The review of input data - currently used input data are shown on the screen. Press P to print the screen.

The review of reports  - results of simulation are shown on the screen. Press P to print the screen.

Note: Reports are stored in files called input and report. These are only temporary files and they are changed every time the simulation is activated. If you need to save them use DOS commands to rename before another simulation run.

Figure 4

simulation clock

macnine number

part number

part type

number of finished parts and buffer status