NavLog 1.1

Aircraft Navigation Aid

(c) 1992 Scott MacLean

NavLog 1.1 Aircraft Navigation Aid              (c) 1992 Scott MacLean

Thank you for trying NavLog!  This is the first  aviation-related

program I have written, and if the response is generally good, it will

be the first in a series.  I have written NavLog to be easy to use for

pilots, who are not necessarily computer experts. To this end,  NavLog

utilizes a GUI (Graphical User Interface) and for that reason requires

that you have:

- A VGA card and monitor, either color or monochrome

- A Microsoft-compatible mouse

NavLog  will  refuse  to run if either of these hardware items is

missing.

History Behind NavLog

When I began flight training,  I was  intrigued  by  the  use  of

analog  "whiz  wheel" E6B computers in solving time/speed/distance and

wind correction problems.  I figured that anything an analog  computer

could do,  a digital computer could do better. I started making cross-

country flights, each one requiring a full navigation log.  These logs

took  a long time to calculate and fill out,  and errors crept in very

easily.  The same calculations were being done on each line,  over and

over  again.  The  complexity  and repetition of the procedures caused

something to click in my mind: numbers,  repetition -  this  task  was

crying  out  for computer automation!  I did some research and found a

book called "The Axioms of Flight," by James  Embree.  This  excellent

book  describes the formulae used in aviation,  with full explanations

and developments for each one. This book helped me develop some of the

formulae used in  NavLog.  I  also  had  some  assistance  from  David

McKeehan.  David is an engineer and a fellow aviation nut from Houston

Texas who helped me with some formulae and beta-testing.

NavLog began as a small character-driven  program  which  allowed

you to input some numbers,  and it would output some other numbers. It

soon became apparent that there just wasn't enough space on an  80  by

25  character  screen to fit all the data that was required.  The next

step was to utilize the  high-resolution  VGA  screen,  and  the  font

capability  of  Microsoft  Basic  7.1 PDS.  It only made sense at this

point to use the mouse as an input device,  so mouse support was added

as well.

Once  the basic version of NavLog was operational,  I started ad-

ding other things. Calculation of pressure and density altitudes, cal-

culation of true and calibrated airspeed,  and more.  Time,  speed and

distance  to  climb  calculations were added from data gained from the

pilot's operating handbook of the Cessna 152 and Cessna  172.  I  will

add more airplanes to NavLog as I fly them!

Finally  it  was  time  to  test NavLog and find the bugs.  David

helped by running it on his Compaq laptop with a math coprocessor. (By

the way, NavLog WILL use the math coprocessor to speed up calculations

if one is installed.) When I was happy with NavLog,  it  was  time  to

release it, and that is what you are looking at right now!

- 2 -

NavLog 1.1 Aircraft Navigation Aid              (c) 1992 Scott MacLean

Using NavLog

When  you  first start NavLog,  you will be shown an introductory

page. When you have finished reading this page, NavLog will build it's

worksheet grid.  You will notice that NavLog sets up the grid to  look

just  like a standard aviation navigation log.  Notice that some boxes

are lighter than others.  You are allowed to modify the numbers in the

light boxes, while NavLog calculates the numbers in the dark boxes.

To  begin  using NavLog,  move the mouse pointer to the box which

you wish to alter. As you move the mouse, the mouse position indicator

on the right hand side of the screen will tell you the name of the box

that the pointer is on. For our example,  we are going to create a log

for a fictional trip from Macon, Georgia to Dublin, Georgia. To begin,

point to the first "Checkpoint" box and click the mouse button. A cur-

sor  will appear in the box,  and you will be allowed to type whatever

you want. Type "Macon" in this box,  and press RETURN to get the mouse

pointer  back.  In  the second box,  type "Top of Climb," in the third

type "Large Road" and in the fourth, type "Dublin Airport." The check-

point boxes do not have any effect on  calculations  (at  this  point,

anyway), so you may type anything you wish in them.

Next,  we  will  need to know what altitude we will be flying at.

Move your pointer to the first altitudes box and press a mouse button.

On the right side of the screen, you will see NavLog asking you for an

indicated altitude.  Type 3500 (this will be our  cruising  altitude),

but do NOT press RETURN.  If you press RETURN at this point, the indi-

cated altitude of "3500" feet will be put into this  box.  Instead  of

pressing RETURN,  press and hold the "ALT" key, then press the "A" key

(for "All").  This will cause NavLog to insert "3500" into  every  al-

titude box downwards. This same "ALT-A" feature works for every box on

the screen except for the checkpoint box.

You  will  notice  that there are actually three altitudes: Indi-

cated,  Pressure and Density.  You are always typing in the  Indicated

altitude. NavLog will automatically calculate the Pressure and Density

altitudes according to the temperature and altimeter settings you have

entered.  Enter  the  temperature  in  FAHRENHEIT only,  in the "WIND"

column.  The altimeter setting for each line is in  the  third  column

from the right, under the "estimated groundspeed" boxes.

Next,  we  need  to enter the wind direction and velocity.  Typi-

cally,  this will be taken from  a  DUAT  or  other  aviation  weather

report.  On the first line, enter the SURFACE winds for your departure

airport. In the remaining wind boxes, enter the "Winds Aloft" data for

your cruising altitude.  Similarly,  the temperature in the first line

should  be  the airport's surface temperature,  and the remaining tem-

peratures should be the winds aloft temperatures.

The CAS/TAS column has an automatic calculation feature.  If  you

enter  a  CAS  value,  NavLog  will calculate the TAS value,  and vice

versa.  This feature only works properly if you have already entered a

proper altitude, temperature and altimeter setting for that line.

- 3 -

NavLog 1.1 Aircraft Navigation Aid              (c) 1992 Scott MacLean

The  next  column is "TC," for True Course.  Enter the course you

plotted on your sectional chart here.  For our example,  the course is

103�. Notice that NavLog will automatically calculate the wind correc-

tion  angle to get your "TH," or True Heading.  Next you need to enter

your magnetic variation.  Remember,  East is Least and West  is  Best.

This  means  if  you are entering an east deviation,  enter a negative

number,  and if you are entering a west deviation,  enter  a  positive

number.  NavLog will calculate your "MH" from this,  or Magnetic Head-

ing. Lastly, enter your compass deviation. This is taken from the com-

pass correction card in your airplane.  It will be added or subtracted

(depending  on  whether  you enter a positive or negative number) from

your MH to get your "CH," or course heading.  This is the number  that

you will actually steer in your airplane when flying the trip.

In  the next column,  you will see the "Distance" entries.  Enter

the distance between checkpoints in each applicable box.  NavLog  will

calculate  the  total distance,  and the remaining distance after each

checkpoint. If you have entered a TAS or CAS,  NavLog will have calcu-

lated an estimated groundspeed. If you have entered winds and courses,

NavLog  takes  these  into  account  when  calculating  the  estimated

groundspeed. If NavLog has calculated a groundspeed, it will also have

calculated an estimated time enroute value.  This time is in hours and

minutes, and is the time estimated for each leg. At the bottom of this

column  is  a  total  time  value,  which  is  also shown in hours and

minutes.

Lastly,  we move onto the fuel column.  At the top of the column,

enter  the  number  of gallons your aircraft burns each hour at cruise

setting.  At the very bottom of this column is the total  usable  fuel

your aircraft is loaded with, in gallons. (Obviously if you are carry-

ing  less fuel due to weight restrictions,  enter the actual amount of

fuel on board here.) Once these values are entered, NavLog will calcu-

late the amount of fuel used in each  leg,  and  the  amount  of  fuel

remaining at the end of each leg. If your trip uses more fuel than you

are carrying,  NavLog will show NEGATIVE fuel remaining,  flagged with

small asterisks (**).

NavLog Functions

On the far right hand side of the  screen  are  seven  functions.

These  functions  are  activated by moving the mouse pointer on top of

them and pressing the mouse button.

SAVE/LOAD

SAVE and LOAD are fairly obvious - they simply save and load your

current worksheet. Standard eight character filenames are used. A good

idea is to use airport identifiers.  For instance,  for  a  trip  from

Macon  to  Dublin,  you  could  call  the  file "MAC-DBN" or something

similar.

- 4 -

NavLog 1.1 Aircraft Navigation Aid              (c) 1992 Scott MacLean

PRINT

NavLog will print your navigation worksheet  on  any  Epson  com-

patible printer.  No fancy high-density,  high-resolution graphics are

used, just the basic EPSON graphics commands.  This should make NavLog

compatible with most printers on the market.  To abort printing, press

the ESC key.

INFO

Info displays a small screen with some extra information  on  the

program, as well as some credits. Press any key (or a mouse button) to

return to your worksheet when finished.

CALC

You may turn the auto-calculation functions of NavLog on and off.

This  applies  only  to  horizontal calculation.  Vertical calculation

(such as calculating total fuel remaining) is  still  done.  Remember!

Whenever the CALC flag is dark, no calculation is being done! For nor-

mal usage, this flag should be on.

TYPE

This button is covered in the next section.

QUIT

Quit  means  what  it says: Quit to DOS.  To ensure that it isn't

selected in error,  you must press BOTH mouse  buttons  simultaneously

while selecting QUIT, otherwise it will not work.

Time/Fuel/Distance to Climb

The  last,  and  most useful function of NavLog is its ability to

calculate the time,  fuel and distance required to climb to cruise al-

titude  from  takeoff.  Because this involves complex calculations and

interpolation based on specific data obtained from an aircraft's  POH,

each aircraft has to be specifically written into the program. At this

time,  NavLog supports the Cessna 152,  172, Piper Arrow III and Semi-

nole.

To switch between the different  aircraft  datasets,  select  the

"TYPE"  button  on  the  right  hand  side.  You  will notice that the

aircraft identifier on the bottom of the screen will switch  from  one

identifier to the next and back again each time you select "TYPE."

To activate the time/fuel/distance to climb function,  we have to

alter the name of the first checkpoint. First,  we must know the field

elevation  of the departing airport.  Enter this elevation between two

slashes, followed by the airport name.  For instance,  Macon is at 463

feet. Therefore, it would be entered as "/463/Macon." When NavLog sees

that the first checkpoint consists of a number enclosed by slashes, it

will  recalculate the first line automatically,  based on a climb from

- 5 -

NavLog 1.1 Aircraft Navigation Aid              (c) 1992 Scott MacLean

field elevation to cruising altitude.  It will calculate the time  re-

quired,  distance  travelled and fuel consumed (including manufacturer

recommended values for run-up and taxi).  See the bottom of this docu-

ment for technical information relating to these values.

Files

This  is  a  listing of the files which are supplied with NavLog,

and what each one does:

NAVLOG.DOC     This file

NAVLOG.EXE     The NAVLOG program itself

HELVE.FON      Font file for NAVLOG

TMSRE.FON      Font file for NAVLOG

CLEAR.CP       "CLEAR" worksheet

CLEAR.NVG      "CLEAR" worksheet

Both .FON files must be present in order for NAVLOG to run. When-

ever a worksheet is saved,  two files are created: a .CP  file  and  a

.NVG file.  The .CP file is 607 bytes long,  and the .NVG file is 1111

bytes long.  Both files are required in order to  load  the  worksheet

again.  The  "CLEAR"  worksheet  which  is  supplied is simply a blank

worksheet which can be loaded to clear out any existing worksheet.

Conclusion

Used properly,  NavLog can replace the tedious job of calculating

navigation logs by hand, using primitive analog computers, and follow-

ing  hard-to-read  charts in the POH.  I have worked steadily for just

over a month on this program,  and sincerely hope it will help you  as

much as it has helped me.  I have shown the output of NavLog to an FAA

examiner,  and even used it for one of my checkrides.  In every  case,

the  numbers  put out by NavLog have been within one to two percent of

the actual values experienced in flight.

In releasing this program to the public, I hope that those people

who find it useful,  and who use it on a regular basis will see fit to

compensate  me  in some way.  Be it a letter with five bucks in it and

some suggestions,  a program you have written yourself,  or perhaps  a

job offer flying your corporation's Lear jet. The amount of response I

get  from  this  program  will  determine how hard I will work towards

writing more aviation-related programs.

I can be contacted at:

Scott MacLean

6 Marilyn Crescent

Georgetown, Ontario, Canada

L7G 1K4

BBS: (301) 353-0593

Thank you for your support, and thank you for using NavLog.

- 6 -

NavLog 1.1 Aircraft Navigation Aid              (c) 1992 Scott MacLean

Technical Documentation

The following information is how the manufacturer  specifies  the

aircraft  must  be  configured  in order for the fuel/time/distance to

climb values inside this program to be accurate.

Cessna 152:         Added December, 1991

From 1980 POH

Flaps up

Full throttle

Mixture leaned above 3000 feet for maximum RPM

Cessna 172:         Added December, 1991

From 1981 POH

Flaps up

Full throttle

Mixture leaned above 3000 feet for maximum RPM

Piper Arrow III:    Added February, 1992

(PA-28R-201)        From 1976 POH

Flaps up

Gear up

Power - 2700 RPM, full throttle

Climb speed - 90 KIAS

2750 lbs gross weight

Piper Seminole:     Added February, 1992

(PA-44-180)         Flaps up

Gear up

Cowl flaps open

Power - 2700 RPM, full throttle, both engines

Climb speed - 88 KIAS

- 7 -