*******************

Supplied functions:

*******************

sio_openport

************

SioPort *sio_openport(int baseAddress, int hardwareInterruptRequestNumber);

Opens a serial port using the specified base port address and irq number.

Returns NULL (0) if any error occured during setup, or a pointer to the

following structure:

typedef struct {

volatile int sio_error;     /* a reveive error has occured */

volatile int sio_errct;     /* number of receive errors */

volatile int sio_chars_sent;/* number of characters transmitted */

volatile char sio_break;    /* a break has been detected */

volatile char sio_linestat; /* current line status */

volatile char sio_modemstat;/* current modem status */

int sio_doxoff;             /* enable s/w flow control */

int sio_brkmode;            /* enable break detection */

} SioPort;

WARNING!!! NEVER,EVER use this structure inside another, or declare a variable

of this type, ONLY EVER use the pointer to this structure returned by

sio_openport.  There are hidden fields critical to the operation of serio.s

that will get corrupted if you ignore this warning. This also means that

`sizeof' will produce bogus results. See serio.s if you want to know the true

layout of this structure (~2kb!).

The port should be closed by sio_closport when it is no longer needed. It does

not matter what order the ports are openned and closed, serio is designed so

that ports that share an irq (if the hardware allows) do not interfere with

any other ports on the same irq.

sio_closeport

*************

void sio_closeport(SioPort *port);

Closes a port previously openned by sio_openport. Order of closure is

unimportant, but all serial ports must be closed on program exit or your

computer will behave unpredictably (may lockup).

sio_put

*******

int sio_put(SioPort *port, char characterToSend);

Attempts to put a character into the transmit buffer. Returns 0 for success

and 1 for failure (buffer full).

Example:

/* transmits an 'X' on com1.

{

SioPort *sioPort=sio_openport(0x3f8,4); /* com 1 standared */

/* initialise port parameters */

while (sio_put(sioPort,'X'))/* optional timeout detection code */;

sio_closeport(sioPort);

}

sio_write

*********

int sio_write(SioPort *port, void *data, int len);

Attempts to place a block of `len' bytes into the transmit buffer. Returns the

number of characters successfully placed into the buffer.

Example:

{

int len,written;

char *buffer;

SioPort *sioPort;

...

while (len) {

written=sio_write(sioPort,buffer,len);

len-=written;

buffer+=written;

}

...

}

sio_senddone

************

int sio_senddone(SioPort *port);

Returns 1 if there are still characters in the transmit buffer, 0 if it is

empty. Supplied by Roland Exler <R.Exler@jk.uni-linz.ac.at>

sio_charready

*************

int sio_charready(SioPort *port);

Returns 1 if there is at least one character waiting in the receive buffer, 0

if it is empty.

sio_get

*******

int  sio_get(SioPort *port);

Returns (and removes) the first character available in the receive buffer.

Blocks if there are no characters available (use sio_charready to avoid

blocking) and returns -1 if a break is detected while waiting for a character

and port->sio_brkmode is non-zero.

sio_sendbreak

*************

void sio_sendbreak(SioPort *port, int charactersToHoldBreak);

Asserts a break condition on the serial line for the time it takes to transmit

`charactersToHoldBreak' bytes.

sio_setspeed,sio_getspeed

*************************

void sio_setspeed(SioPort *port, int baudRateDivisor);

Sets the speed of the serial port to the specified rate.  DOES NOT convert

from buad (may be a non standard port).  For standard ports use:

sio_setspeed(sioPort,115200/baud_rate).

int sio_getspeed(SioPort *port);

Returns the current setting of the serial port speed. DOES NOT convert to baud

(may be a non standard port).  For standard ports use:

baud_rate=115200/sio_getspeed(sioPort).

sio_setmcr,sio_getmcr

*********************

void sio_setmcr(SioPort *port, int valueToSetModemControlRegisterTo);

int  sio_getmcr(SioPort *port);

These functions allow direct control of the modem control register. Not

normally used.

sio_setlcr,sio_getlcr

*********************

void sio_setlcr(SioPort *port, int valueToSetLineControlRegisterTo);

int  sio_getlcr(SioPort *port);

These functions allow direct control of the line control register. Not

normally used.

sio_setparms

************

void sio_setparms(SioPort *port,

enum sioWordSize,

enum sioParity,

enum sioStopBits);

Sets up the `formatting' of the serial port. The available settings are:

enum sioWordSize {

sio5Bits,

sio6Bits,

sio7Bits,

sio8Bits,

};

enum sioParity {

sioNoParity     =0x00,

sioOddParity    =0x08,

sioEvenParity   =0x18,

sioMarkParity   =0x28,

sioSpaceParity  =0x38,

};

enum sioStopBits {

sio1StopBit=0x00,

sio1_5StopBits=0x04,

sio2StopBits=0x04,

};

**************************

Locking the code and data:

**************************

Due to the nature of DPMI and the fact that serio is interrupt driven, you

must lock down all code and data touched by serio.s.  This can be done by

including the following code somewhere in your code BEFORE sio_openport is

called for the first time.  It needs to be called only once.

/* Required by BCSerio */

extern char sltext[] asm ("sltext");

extern char eltext[] asm ("eltext");

extern char sldata[] asm ("sldata");

extern char eldata[] asm ("eldata");

_go32_dpmi_lock_code(sltext,eltext-sltext);

_go32_dpmi_lock_data(sldata,eldata-sldata);

My recomendation is to do the following:

static void __attribute__((contructor))lock_sections(void)

{

extern char sltext[] asm ("sltext");

extern char eltext[] asm ("eltext");

extern char sldata[] asm ("sldata");

extern char eldata[] asm ("eldata");

_go32_dpmi_lock_code(sltext,eltext-sltext);

_go32_dpmi_lock_data(sldata,eldata-sldata);

}

lock_sections will be called before main by the libc startup code.

The supplied djgpp.djl is a linker script (for ld) required to get the locked

data and text sections to work.

***********

Known Bugs:

***********

There are no KNOWN bugs in serio, however, it does still have some problems:

o It doesn't work under windows 3.x due to the bug in W3.x's dpmi server that

causes interupts to be lost after a `cli' instruction.  I have a few ideas

about how to get around this, but I haven't gotten around to it yet, nor the

motivation as I no longer use 3.11 (work's upgraded me to w96).

o To use under windows 95, it seems you have to disable the desired serial

port, otherwise serio.s hangs while trying to reset the port.

o There is a bug in gas (versions upto and including 2.8.1) that caused it to

have problems with code sections thar are not called `.text'. I have a patch

(for version 2.8.1) to fix this and I beleive Robert Hoehne

<robert.hoehne@mathematik.tu-chemnitz.de> will be incororating (sp?) the patch

into the djgpp port of binutils 2.8.1. For this reason, serio.o is supplied

pre-compiled.  If you need the fixed as.exe, send me some email.

o Not a bug, but a gotcha:  when linking serio.o into your code, you must use

the supplied linker script (either in the current directory while linking, or

replace the one in $(DJDIR)/lib (it's fully compatable)) otherwise all the

interrupt and interface code will not be linked in correctly.

Bill Currie <billc@blackmagic.tait.co.nz>