Loading the ODI drivers at an interrupt other than 60.

ODIPKT # int

|  |

|  +-- desired interrupt vector, in decimal (e.g.,

|      0x60h=96, 0x65h=101

|

+----- logical board number (determined via NET.CFG),

one of 0,1,2 or 3.

E.g., You have an SMC Ethercard Elite, want to use interrupt 62 (hex)

and the following NET.CFG:

link driver SMCPLUS

port 240

frame Ethernet_802.3

frame Ethernet_II

[...]

You would load ODIPKT thus:

ODIPKT 1 98

You specify logical board 1, because in your NET.CFG, Ethernet_II is

set-up as the second logical network board (Ethernet_802.3 is the

first logical board).  If you're only using Ethernet_II packets, you

specify logical board 0.--

_________________________________________________________________________

RUCS     | Mark A. Medici, Systems Programmer III, User Services Division

User     | Rutgers University Computing Services, New Brunswick, NJ 08903

Services | [medici@gandalf.rutgers.edu]                    [908-932-2412]