STANDARD MICROSYSTEMS CORPORATION

Packet Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994-1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Basic packet driver use.

o  Software interrupts.

o  How to use the packet driver to connect to a NetWare 2.2 server

using the BYU packet driver shell.

o  How to use the packet driver to connect to a NetWare server

using the Intel packet driver shell.

o  Using the packet driver with SMC adapters.

Note: This driver supports all 8432, 8432 (Enhanced), and 9332 Ethernet

adapters.

BASIC PACKET DRIVER USE

The following section describes basic packet driver use. Since

different networks have different requirements, you can include the

following switches in your command lines as needed.

-d    This switch delays the adapter's initialization until the first

time the packet driver is accessed. Use this switch if your

workstation loads the operating system over the network using a

Boot ROM on the SMC adapter.

-n    This switch configures the driver to change 8137-formatted

packets seen by the client into IEEE802.3 packets.  Unless you

use the BYU's NetWare packet driver shell, you can omit this

switch. If you want to use 8137 packets, use "econfig" to

configure Intel's PDIPX103 packet driver shell.

-p    This switch disables promiscuous mode. A small level of security

can be achieved by disabling promiscuous mode; however,

disabling promiscuous mode should not be mistaken for real

security.

-w    This switch is used with Microsoft Windows for nonresident DOS

TCP stacks, such as NCSA Telnet and PC-Gopher. Alternatively,

you can use the Windows freely copyable utility, winpkt.

-i    The SMC packet driver can be used with both Ethernet_II and

IEEE 802.3 frame types. The packet driver specification, however,

only allows a driver to report one class which, by default, is

Ethernet_II. If you use the packet driver with client software

that expects to find an IEEE 802.3 packet driver instead of an

Ethernet_II packet driver, use this switch to configure the

driver to use IEEE 802.3-formatted packets.

SOFTWARE INTERRUPTS

A packet driver provides services for client programs. These programs

access the packet driver by issuing software interrupts.

Any software interrupt between the hexadecimal addresses of 0x60

through 0x80 can be used with the packet driver, with the following

exceptions:

o  0x67 is reserved for the EMS specification.

o  0x70 through 0x77 are reserved for hardware IRQ lines 8 through 15.

After you choose a software interrupt, assign that interrupt to the

packet driver, and install the packet driver. The client programs

will then automatically locate the packet driver by searching for

the packet driver's signature at software interrupts from 0x60

through 0x80.

MEDIA/SPEED KEYWORDS

Following the optional slot number parameter are several optional

keywords.  If you specify media then you must specify speed (and

vice-versa).  These select the media:

AUTONEGOTIATE - (Negotiated -- default)

MII - Media Independent Interface

UTP - Unshielded Twisted Pair

BNC - British Naval Connector (thinwire)

AUI - Attachment Unit Interface (thickwire)

S10 - Starlan 10 (pre-standard 10Base10 -- no link beat)

Other keywords select the speed.  FOUR and SIXTEEN are for Token

Ring.  TEN and HUNDRED are for Ethernet.  FD is for full duplex

interfaces.

INSTALLING THE PACKET DRIVER

The command line has the name of the packet driver, any dash options

(as listed above), the software interrupt, and any optional

parameters, such as slot number, or media/speed keywords.  The

simplest command line is "pktpwr 0x7e".  A more complex line would be

"pktpwr 0x7e aui", or "pktpwr 0x7e hundred fd".

CONNECTING TO A NetWare 2.2 SERVER USING THE BYU PACKET DRIVER SHELL

If you want to connect to a NetWare 2.2 server that, by default, is

configured to use IEEE 802.3-type packets, use the following command

line examples.

PKTPWR -n 0x7e

BYUIPX

NETX

In this command line:

o  PKTPWR denotes the SMCPower adapter.

o  -n configures the driver to change 8137-formatted packets seen by

the client into IEEE802.3 packets.

o  Software interrupt setting 0x7e is the hexadecimal address 7E,

which allows packet driver clients to use interrupt 7E to

communicate with the packet driver.

CONNECTING TO A NetWare SERVER USING THE INTEL PACKET DRIVER SHELL

If you want to connect to a NetWare server using the Intel packet

driver shell, pdipx103, use the following command line:

PKTPWR 0x7e

PDIPX

NETX

In this line:

o  PKTPWR denotes the SMCPower adapter.

o  0x7e is the hexadecimal address 7E, which allows packet driver

clients to use interrupt 7E to communicate with the packet driver.

USING THE PACKET DRIVER WITH TWO SMCPower ADAPTERS IN ONE MACHINE

You can use the packet driver with two adapters in the same machine.

The first packet driver will use the first adapter found, which will

be in virtual slot number 16, the first PCI slot.  The second packet

driver should specify virtual slot number seventeen, which is the

second PCI slot.

pktpwr 0x7d

pktpwr 0x7e 17

In these lines:

o  PKTPWR denotes the SMCPower adapter.

o  0x7d is the hexadecimal address 7D, which allows packet driver

clients to use interrupt 7D to communicate with the packet driver.

Similarly for 0x7e.

o  17 is the Virtual slot number.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.