Each field is a sperate entity including the

H addresss......it is seached from right to left.....

so if the address is:

SB all @ xyz.#subregion.State.Country.Continent

This will be scanned or parsed from the continental designator towards

the left until it finds a match.....

so if the address was

SB all @ MN.USA.NA

it would first look at at NA and disregard it because I am already in

in the domain of NA. The same with USA. I'm not in MN, so that will

be used for forwarding.

Now the address can also be broken down so that SB all @ allus would work to

without any H address...this is called a distributor and would fwd based on

ALLUS to specific callsigns as set in the dist.mb file....these type of flooders

do not go in the fwding file and are not attached to any Hroute as in FBB.

So the main result is that my fwding file information contains the following

information based on domains.

1> if the @ bbs field is a local system it forwards based on the @bbs

field....ie sp n6xyz @ wa6rdh.#nocal.ca.usa.na would forward on wa6rdh

as he is in my domain.

2> If the @ bbs field is not within my local domain but adjactent domains

within the next biggest domain then the sub regional desigantor is used

for fwding.....ie sp n6xyz @ w6ll.#cenca.ca.na.usa the #cenca is used

for fwding..

3) the Domain of CA is completely ignored as I am in that domain and the

local bbs plus the subregions are also within that domain.

4) If the @bbs field is not known and the subregional domain is not known

and the state domain is known then it is used.

e.g. sp k0zxf @ k0zxf.#tpa.fl.usa.na the fl is used for fwding.....

5) If the domain is outside of the 50 states, but still in NA,

then the country domain is used.

6) If the domain is outside NA then the continental domain is used.

7) Flooders are used to distribute to to specific direct stations.

The hierachical location information is ignored.

mrybay - my lan

allcan - northern calif

allca  - all of calif

allusw - Western US

allus  - all of the US

ww	  - the world

These are directed to specific direct connect stations as designated

in my dist.mb file which is a very small list.....they are then responsible

for fwding this to the next station in the line, etc etc etc.

8> Now what this system fails to deal with is how we address messages to

domains other than our own....hence the bulletins for addressing that

I have sent out in the past to deal with the issue.....which is what

you are alluding to in your request....yes we can use the H address

to send traffic and address it to specific domains without problems.

For example:

SB all @ FL.USA.NA will work so will SB all @ fl.fl.usa.na

SB all @ CAN.NA will work

SB all @ EU.EU or SB all @ GBR.EU will work

all will forward based on the domains that the system knows about.

I did propose that we switch to such a system as did Brian, KA2BQE at

one of the digital conferences.....based on the following:

Local     - @ bbs callsign

Regional  - Designator beginning with the '#' character.

State     - State, Province, Department, Prefecture, etc.

ISO designator where available (e.g. CA, FL, OR)

otherwise abbreviation in common use in the country.

Country   - 3 letter ISO designator.

Continent - 2 letter ISO or 4 letter w3iwi designator.

All that needs to be in your fwding system are the callsigns of other bbs

in your Region, regional designators within your state (except for the

Region you are in), state domains (except for the state you are in),

country designators within your Continent (except for the Country you

are in), and the Continetal names for continents other than your own.

Very simple and does not require a raft of callsigns or designators.