Updated 02-23-90

DSZ can work with non-standard IRQs.  I have included here a  descrip-

tion of the "portx" feature of DSZ.    I now have DSZ working on  COM2

in a non-standard configuration.    It is important that the port # be

assigned not only in the PROTOx.DEF  ( Note:  My DEF file for NODE2 is

PROTO2.DEF. ) file but also by using Ray Guin's X00.SYS as seen in the

example below.

To support specialized  multiport serial interfaces, the portx command

will  select a  communications  port with a specified hexadecimal base

address hbase and Interrupt Request Line IRQ.  The portx command won't

check its argument  for  validity.    Chuck Foresberg indicates in his

documentation  for  DSZ  that  "this command is not available on X.PC,

Unix, Xenix, or OS/2."  I  am  running  RBBS on a IBM-PC/XT with an AT

expansion case and a Tandy 1000TL in a LAN using LANtastic with Ether-

net Cards in both machines.  Since my Ethernet Card in the XT is using

IRQ3 I have  had to configure COM2 to use  IRQ2.   I have two nodes on

line which are running  very  smoothly  with this  configuration using

PUMA  and  DSZ  as external protocols.   Multitasking is managed under

DesqView.  Note:  Foresberg notes that "IRQ2" will not work as  a  COM

Port Interrupt on a IBM-PC/AT because of Disk and Video Interrupt con-

flicts.

With the  combination  of Ray Gwin's X00.SYS and the portx feature of

DSZ  RBBS-PC can be run with  non-standard communication port config-

urations  along with the several popular protocols provided by DSZ in

addition to RBBS's internal protocols.

PROTO2.DEF EXAMPLE:  "port[PORT#] 2f8,2"  Selects a comm port with a

base address of 2f8, using IRQ2.

CONFIG.SYS EXAMPLE: DEVICE=X00.SYS 0=3F8,IRQ4 1=2F8,IRQ2

( In this configuration COM1 is standard COM2 is non-standard )

( Read  X00.DOC for  further  configuration  specifications.  )

RBBSxPC.DEF:  Parameter 233 must have correct PROTOx.DEF identified.

For further reference John Taylor's DesqView documentation in RBBS-PC

documentation further explains interrupts for COM ports and hexideci-

mal addressing. Or RBBS-DV2.ZIP contains his remarks on this topic as

well.

Tom Hansen

Church Chatters (RBBS-PC CPC17.3/"MiteSized)

24 hrs. 3/12/24/9600

DATA: ( 308 ) 872-3394