�����������������������Ŀ

��The Batch File Enhancer�

����������DO Shareware version 1.31��������Ŀ

�Copyright (C) 1990-95 EaglePlus software...�

��������ĿWritten by, Adeeb Mass'ad����������

��ĿAll rights reserved����

���������������������

TABLE OF CONTENTS

[A] - DO ....... The Batch file enhancer

[B] - DO ....... Example files

[C] - DO ....... Why should I register?

[D] - DO ....... Appendix A

[E] - DO ....... Appendix B

[A] - DO v1.30

--------------

Syntax: DO [Command...]

Purpose: This program is a batch file enhancer. That will help you

to improve your batch file performance with a brand new

look & feel. It contains a lot of useful commands.

Remarks: Command is one of the followings:

��������������������������������������������������������������������Ŀ

� No.� Command          � Parameters                                 �

��������������������������������������������������������������������Ĵ

�  1 � Window           � X1 Y1 X2 Y2 [WinClr] [WinBack] [WinShadow] �

�  2 � SetBorder        � [BorderColor]                              �

�  3 � SetPalette       � <Color> <Palette value>                    �

�  4 � Clear            � [BackColor]                                �

�  5 � Fill             � "Char" [TextClr] [BackClr] [X1 Y1 X2 Y2]   �

�  6 � Prt              � TextClr BackClr ["String"]                 �

�  7 � PrtLn            � TextClr BackClr ["String"]                 �

�  8 � Goto             � X Y                                        �

�  9 � Sound            � SndNum Delay [Wait] [Repeat]               �

� 10 � Sound            � /filename [KEYSTOP]                        �

� 11 � Mode             � [Co80/Co40/Bw80/Bw40/Mono/EGON/EGOF/Reset] �

� 12 � Reboot           � [(C)old or [(W)arm]][(V)erify]             �

� 13 � Rem              � Comment                                    �

� 14 � GetKey           � list of keys                               �

� 15 � GetMemSize       � [check size in KB]                         �

� 16 � CheckSpace       � (size in KB) (driveletter)                 �

� 17 � CheckEga         �                                            �

� 18 � Pause            � Pauses utill a key is pressed.             �

� 19 � /filename        � Inputs from filename                       �

����������������������������������������������������������������������

[1] - Window X1 Y1 X2 Y2 [WinClr] [WinBack] [WinShadow]

Example: DO Window 10 10 40 20 Brown Cyan Red

Draws a window from 10,10 to 40,20 with brown border color

and cyan background and a red shadow. (look at appendix A

for colors)

[2] - SetBorder [BorderColor]

Example: DO SetBorder Blue

This command is just like the utility BORDER.EXE. It

sets the border color, but the colors are in words.

The border color using this command can take effect

on CGA, EGA or higher adapters.

[3] - SetPalette <Color> <Palette value>

Color = Color name

Palette Value = EGA palette value 0-63

Example: SetPalette Blue 8

Changes the value of the Blue color, this color

Darken the blue color.

For a clearer picture about the EGA palette valaues

try to get our EGA Palette Manager v1.10 or higher.

(PAL110, soon PAL120 will be released)

Note:  The color name Border can be used with this function.

It will change the palette value of the Border Color.

[4] - Clear [BackColor] [Foreground]

Example: DO Clear

Clears the screen.

DO Clear Brown

Clears the screen with Brown background.

DO Clear Black LGray

Clears the screen with Black background &

Light-Gray foreground.

[5] - Fill "Char" [TextClr] [BackClr] [X1 Y1 X2 Y2]

Example: DO Fill "*"

Fills the whole screen with * & using the current color.

DO Fill "*" Yellow Blue

Fills the whole screen with * using Yellow for the * color

and Blue for it's background.

DO Fill "*" Yellow Blue 40 1 80 25

Like the last but this time fills the screen from 40, 1 to

80, 25.

[6] - Prt TextClr BackClr ["String"]

Example: DO Black White "Hello Batch file!"

Prints using Text & Background color.

[7] - Prtln TextClr BackClr ["String"]

Example: DO Yellow Blue "How are you?"

Acts like PRT but starts new line.

[8] - Goto X Y

Example: DO Goto 10 5

Sets the cursor at 10, 5.

[9] - Sound SndNum Delay [Wait] [Repeat]

SndNum - Sound code (you must know these codes to make music)

Delay  - Delay in milliseconds.

Wait   - Wait in milliseconds.

Repeat - How many times to repeat the note.

Example: DO Sound 130 100

Plays note C, length 100 milliseconds.

DO Sound 130 100 50 2

Like the last but waits 50 milliseconds after the note and

repeats it two times.

[10] - Sound /filename [KEYSTOP]

Example: DO Sound /Song1.DO

Plays the notes that from Song.Do.

DO Sound /Song1.DO KEYSTOP

Plays Song1.DO & stops if key pressed.

[11] - Mode [Co80/Co40/Bw80/Bw40/Mono/EGON/EGOF/RESET]

Example: DO Mode

Stores the Mode number in Errorlevel.

DO Mode Reset

Resets the current video mode. If you were for example

in Color 80x25 video mode, this function will take the

same effect as DO Mode Co80.

DO Mode Co80

Changes the text mode to Color 80x25.

(look at appendix B)

[12] - Reboot [(C)old or [(W)arm]][(V)erify]

C - Cold Boot. The same as turnning your PC OFF then ON, or

using the Reset button.

W - Warm Boot. Or software boot, the same as holding down

Alt+Ctrl then Del.

V - Asks the user to verify the act.

Example: DO Boot W

Makes a Warm Boot.

Note:  Before booting this function will flush all of

SmartDrv's write-behind data.

[13] - Rem Comment

This command has no effect it just for adding notes.

Example: DO Rem This part will draw a big box!

[14] - GetKey keys

Example: DO GetKey MCDQ

Waits for a key M, C, D,  Q then stores it's value

in errorlevel. (M as 1, C as 2, D as 3, Q as 4)

If you pressed [Esc] errorlevel will take 5, in this

example.

[15] - GetMemSize [check size in KB]

Example: DO GetMemSize

Stores the system memory size in the errorlevel. (Dos memory)

DO GetMemSize 56

Checks if more 56KB can fet in your computer's memory.

Errorlevel will return 1 if fets & 0 if not.

[16] - CheckSpace (size in KB) (driveletter)

Example: DO CheckSpace 20 B:

Checks if more 20KB can fet into drive B:.

Errorlevel will return 1 if fets & 0 if not.

[17] - CheckEga

Example: DO CheckEGA

This function checks the presents of an EGA or higher

system, and returns an ErrorLevel 1 if present and 0

if not.

[18] - Pause

Example: DO Pause

Waits for anykey.

Errorlevel will take the key code.

[19] - /filename

Example: DO /Examp.DT1

Read the commands from Examp.DT1.

[B] - DO Example files

----------------------

[1] - CHKSP.BAT

Syntax: CHKSP.BAT (size in KB) (drive letter)

Remarks: This batch file shows how to use the CheckSpace function.

Example: CHKSP 10 B:              - Checks if 10 KBs can fit into drive B:.

[2] - COLORS.BAT

Syntax: COLORS

Remarks: Demonstrates the use of BackGround colors.

Note: Uses one data file. (COLORS.DAT)

[3] - DOMODE.BAT

Syntax: DOMODE or DOMODE [Co80/Co40/Bw80/Bw40/Mono/EGON/EGOF/Reset]

Remarks: Demonstrates the use of Mode functions.

Example: DOMODE                   - Prints the current text mode.

DOMODE Co80              - Go to text color mode 80x25.

[4] - EXAMP.BAT

Syntax: EXAMP

Remarks: Demonstrates the use of several commands, to build a powerful

menus using DO.

Note: This batch file uses four data files EXAMP.DT1/DT2/DT3/DT4.

[5] - GETMEM.BAT

Syntax: GETMEM or GETMEM [MemoryAmount]

Remarks: Demonstrates the use of GETMEM function.

Example: GETMEM                   - Shows your system's dos memory.

GETMEM 100               - Checks if 100KB can fit.

[6] - SONG.BAT

Syntax: SONG

Remarks: Demostrates the use of DO's video & sound functions.

Note: Uses four data files. (SONG.DAT, SONG1.DO, SONG2.DO, SONG3.DO)

[C] - Why should I register?

By your registration you will get better version of DO,

more commands and better documentation.

This version is shareware, you can use it for your own needs.

You cannot build batch files & distribute them with DO.EXE.

After a trial period of 31 days, you are expected to pay the

registeration fee.

When you register you will get a commercial version, that you can

include it with the batch files you write & distribute. (You can

impress your users by menuing your programs, or by building a

colorful installation batch files)

The commercial version includes more useful commands. (We listen to

our registered users, if you want us to add a new command to DO.EXE

your suggestions will be welcomed)

(Sound Blaster/VGA support is under development)

[D] - Appendix A

----------------

This is a table of DO.EXE legal colors.

Color   � Color

������������������

Black   � DGray

Blue    � LBlue

Green   � LGreen

Cyan    � LCyan

Red     � LRed

Magenta � LMagenta

Brown   � Yellow

LGray   � White

D for Dark

L for Light

[E] - Appendix B

----------------

����������������������������������Ŀ

�No.� Mode � Meaning     � Adapter �

����������������������������������Ĵ

�0  � Bw40 � B&W   40x25 � CGA     �

�1  � Co40 � Color 40x25 � CGA     �

�2  � Bw80 � B&W   80x25 � CGA     �

�3  � Co80 � Color 80x25 � CGA     �

�7  � Mono � Monochrome  � HGC     �

������������������������������������