Three COMs

(PC Magazine Vol 6 No 5 Mar 10, 1987 User-to-User)

These three small assembly programs help to smooth out three

separate DOS problems.

(1) If you use your PC's floppy disk drives frequently, you

probably spend a lot of time waiting for the disks to stop spinning

so you can safely switch them.  The memory-resident FASTOFF.COM program

turns off the drive motor much more quickly than the standard IBM PC

does.  Normally the motors are turned off 37 ticks (about 2 seconds)

after the last disk access.  FASTOFF reduces this time by decrementing

the elapsed time counter twice as fast as normal.

A single byte located in the ROM data area is used to maintain

the number of ticks remaining before the motor will be turned off.

Part of the system timer's job is to decrement this byte and turn off

the drive motor when the counter reaches zero.  FASTOFF uses the timer

tick interrupt, which occurs 18.2 times each second, to decrement this

same counter.  Since this program addresses the ROM-BIOS data area,

it will work on an IBM or a very close compatible only.

(2) It seems that after every print screen, the next step is

always to take the printer off-line and press the form feed to eject

the page.  The memory-resident FFEED.COM program will add a form feed

character automatically after the print screen operation is complete.

Since at times the form feed is undesirable, FFEED adds it only if the

Right Shift key was used with PrtSc.

It works by intercepting interrupt 5, which occurs whenever the

keyboard detects a Shift-PrtSc.  It then tests the keyboard status to

see which shift key is being held down.  If it is the right one, the

FF character is sent to the printer when the screen dump completes.

FFEED is also smart enough to check to see if the printer is

on-line and ready before beginning the screen print.  If the printer

is not ready, the screen dump is aborted to prevent the keyboard from

locking up.  This is done by making a printer status call before giving

control to the ROM routine.  It's odd that such a simple check was not

included within the standard ROM routine.

(3) For a young child or an inexperienced typist, the auto-repeat

feature of most keyboards may cause lots of unwanted duplicate

keystrokes.  The memory-resident NOREPEAT program eliminates the

auto-repeat no matter how long a key is held down.

It works by using interrupt 9 to examine each keystroke.  Normally

this detects when each key is depressed and when it's released.  When

a key is held down long enough to repeat, the keyboard simulates the

key's being depressed again and again without being released.  NOREPEAT

senses these unwanted duplicate keystrokes and then prevents them from

being inserted into the keyboard buffer.  When the key is finally

released, new keystrokes are again allowed to occur normally.

###################################################################################################################