This is a early version of a GameGear doc. This is what I know as of now.

Within the next few days I'll hopefully have a sprite under joystick control.

I'm researching setting up a vertical retrace interrupt.

PORT.7Eh:        Current Scanline Counter

PORT.7Fh:        Sound Register. Probably same as TMS9918a sound port, but

haven't checked.

PORT.BEh:        VRAM data WRITE

--------

Output data 1 byte at a time. Low byte first. High byte next.

PORT.BFh:        VDP status (Interrupt,Spr Conc,5th), VRAM address setup, and

Register write

--------

Input from port bfh:

bit  Name  Expl.

0-3  5/9th Number for the 5'th sprite (9'th in screen 4-8) on a line

5    C     1 if overlapping sprites

6    5D    1 if more than 4 sprites on a horizontal line (9 in screen 4-8)

7    F     Copy of the CPU interrupt (1=interrupt)

Output to port bfh:

Register Write: (Write register data. Write $80+register number)

VDP REGISTER LIST:

-----------------

Register 0:

Mode register 0

bit  Name  Expl.

0    D     External video input (0=input disable, 1=enable)

1    M3    Mode M3

2    M4    Mode M4

3    M5    Mode M5 (Always 0)

4    IE1   Horizontal Retrace Interrupt Enable

5    IE0   Vertical Retrace Interrupt Enable

6    DG    Digitize mode

7    0     (Always 0)

Register 1:

Mode register 1

bit  Name  Expl.

0    MAG   Sprite zoom          (0=x1, 1=x2)

1    SZ    Sprite size          (0=8x8, 1=16x16)

2    0     (Always 0)

3    M2    Mode M2

4    M1    Mode M1

5    IE2   Interrupt control (50Hz (PAL), 60 Hz (NTSC)

(0=Disable, 1=Enable)

6    BLK   Screen output control        (0=Disable, 1=Enable)

7    416   VRAM size control            (0=4K, 1=16K)

Register 2:      (Not sure if the register is valid)

Name table base address

Bit     7    6    5    4    3    2    1    0

Name    0   A16  A15  A14  A13  A12  A11  A10

Register 3:      (Not sure if the register is valid)

color table base address Low

Bit     7    6    5    4    3    2    1    0

Name   A13  A12  A11  A10  A09  A08  A07  A06

Register 4:      (Not sure if the register is valid)

Pattern generator base address

Bit     7    6    5    4    3    2    1    0

Name    0    0   A16  A15  A14  A13  A12  A11

Register 5:      (Not sure if the register is valid)

Sprite attribute table base address Low

Bit     7    6    5    4    3    2    1    0

Name   A14  A13  A12  A11  A10  A09  A08  A07

Register 6:      (Not sure if the register is valid)

Sprite pattern generator base address

Bit     7    6    5    4    3    2    1    0

Name    0    0   A16  A15  A14  A13  A12  A11

Register 7:

color register. (Not sure if the register is valid)

bit  Name  Expl.

0-3  TC0-3 Background color

4-7  BD0-3 Foreground color

The bits 0-3 and 4-7 can hold a number in the range of 0-15.

The corresponding colors are:

0 = Transparent         8 = Medium red

1 = Black               9 = Light red

2 = Medium green        10= Dark yellow

3 = Light green         11= Light yellow

4 = Dark blue           12= Dark green

5 = Light blue          13= Magenta

6 = Dark red            14= Gray

7 = Cyan                15= White

Register 8:

Mode register 2  (Not sure if the register is valid)

bit  Name  Expl.

0    BW    32 Grey levels.

1    SP    ? Sprite             (0=On, 1=Off)

2    VR0   VRAM size and speed

3    VR1   VRAM size and speed

4    CB    color Bus direction         (0=Output, 1=Input)

5    TP    ?Transparent from palette

6    LCS   Lightpen Select (active 1) connected trough colorbus

7    MSE   Mouse select (active 1) connected trough colorbus

Register 9:

Mode register 3

bit  Name  Expl.

0    DC    DLCLK Dot Clock Direction    (0=Dot clock Input, 1=Output)

1    NT    NTSC                         (0=60Hz, 1=50Hz)

2    E0    Even Odd                     (0=Normal, 1=Two screen)

3    IL    Interlace                    (1=On, 0=Off)

4    S0    Simultaneus mode

5    S1    Simultaneus mode

6    0     (Always 0)

7    LN    Vertical higth (pixels)      (0=192, 1=212)

Simultaneus mode:   S0  S1  Expl.

0   0   Normal (Intern)

0   1   Mix

1   x   External(Digitize)

Register 0Ah:    (Not sure if the register is valid)

color table base address High

Bit     7    6    5    4    3    2    1    0

Name    0    0    0    0    0   A16  A15  A14

Register 0Bh:    (Not sure if the register is valid)

Sprite attribute base address High

Bit     7    6    5    4    3    2    1    0

Name    0    0    0    0    0    0   A16  A15

VRAM Write Setup: (Write LB of VRAM address. Write $40+HB of VRAM address)

CRAM Write Setup: (Write color number. Write $c0. Write color value LB then HB)

Color number is 0-15 for screen tiles and 16-31 for sprites.

Color Value:

LB - G G G 0 R R R 0

HB - 0 0 0 0 B B B 0

JOYSTICK:

--------

PORT 00h GameGear Start Button

bit   Expl.

0 = Not Used

1 = Not Used

2 = Not Used

3 = Not Used

4 = Not Used

5 = Not Used

6 = Not Used

7 = Start Button

PORT DCh Joystick Port

bit   Expl.

0 = Joystick UP                 (0=Moved, 1=Not moved)

1 = Joystick DOWN               (0=Moved, 1=Not moved)

2 = Joystick LEFT               (0=Moved, 1=Not moved)

3 = Joystick RIGHT              (0=Moved, 1=Not moved)

4 = Joystick trigger A

5 = Joystick trigger B

6 = Not Used

7 = Not Used

PORT DDh Joystick select

bit   Expl.

0 =   Not Used

1 =   Not Used

2 =   Not Used

3 =   Not Used

4 =   Not Used

5 =   Not Used

6 =   Joystick select for SMS   (0=Connector 1, 1=Connector 2)

7 =   Not Used

SCREEN DESCRIPTION. (Screen 2 seems to be the only screen.)

------------------

SCREEN 2 (256*192 Graphics mode. Set by storing $06 to VDP register 0):

Character patterns              0000-37FF       (See note 1)

Name table (char positions)     3800-3EFF       (See note 2)

Sprite attribute table          3F00-3FFF       (See note 3)

Sprite character patterns       0000-37FF       (See note 4)

Font (Note 1):

Byte (+offset):         Expl.:

0-7                Pattern number 0 (1st Plane. Lowest bits of each pixel).

0:      00000000

1:      00001000

2:      00010100

3:      00100010                = "A"

4:      00111110

5:      00100010

6:      00100010

7:      00000000

8-F                 Pattern number 0 (2nd Plane.)

10-17                Pattern number 0 (3rd Plane.)

1F-20                Pattern number 0 (4th Plane. Highest bits of each pixel)

...

Character Patterns (Note 2):

Byte (+offset):         Expl.:

0                  Tile value. Max 224.

1                  Tile attribute. (Still working on this.)

...

Sprite attribute (Note 3):

Byte (+offset):         Expl.:

0-3                Sprite 0 attribute

0:      Vertical position

1:      Horizontal position

2:      Pattern number

3:      Bit:    Expl.:

0-3     Color code

4-6     0

7       EC (Early Clock, 1=> Shift sprite 32 pixels

to the left)

4-7                Sprite 1 attribute

8-B                Sprite 2 attribute

...

Sprite character patterns (Note 4):

Used like the 8*8 pixel Font in screen 0 when using 8*8 pixel sprites.

When using 16*16 pixel sprites, four blocks of 8*8 pixels is organised like:

Byte (+offset):         Expl.:

0-7                8 Bytes, Block A

8-F                8 Bytes, Block B

10-17               8 Bytes, Block C

18-1F               8 Bytes, Block D

And the sprite will be assembled on screen like this:

+---------+---------+

|         |         |

| Block A | Block C |

|         |         |

+---------+---------+

|         |         |

| Block B | Block D |

|         |         |

+---------+---------+

Example Routines:

----------------

Routine to write a value to a VDP register.

a contains the value to be written to the VDP register.

b contains the VDP register to write to.

write_register:

out     (0bfh),a        ; Send data (note: the data is send FIRST)

ld      a,b             ; Get the VDP register in [a]

or      10000000b       ; Set bit 7 to indicate register WRITE

out     (0bfh),a        ; Select register

ret

Routine to Setup VRAM Read/Write Access and read a byte.

hl coutains the VRAM address.

vpeek:  di

call    setup_read_address      ; Setup VRAM read start address

in      a,(0beh)                ; Get byte from VRAM to [a]

ret

Setup_read_address:

ld      a,l

out     (0bfh),a   ; Send Low Byte to VDP

ld      a,h        ; Note: no special bit is set to indicate READ

out     (0bfh),a   ; Send High Byte to VDP

ret

vpoke:  di

call    setup_write_address     ; Setup VRAM write start address

out     (0beh),a                ; Write byte in [a] to VRAM

ret

Routine to Setup VRAM Read/Write Access and write a byte.

hl coutains the VRAM address.

Setup_write_address:

push    af                      ; Save byte in [a]

ld      a,l

out     (0bfh),a                ; Send LOW byteto VDP

ld      a,h

or      01000000b               ; Set bit 6 in High byte to

; indicate WRITE

out     (0bfh),a                ; Send MSB to VDP

pop     af

ret

Note that once set, the VRAM READ/WRITE address will be incremented after each

byte is read from the VRAM DATA READ port.