HANDICAPPED EDUCATIONAL EXCHANGE

11523 Charlton Drive

Silver Spring MD 20902

Computer: (301) 593-7033                        MCI Mail: "RBARTH"

Home:     (301) 681-7372   January 7, 1985      TELEX: 650 248-8912

TDD56

This software permits the IBM-PC and PC-Jr to function as TDDs,  or

Telecommunications Devices for the Deaf. These devices are also known as

TTYs.  This  (TDD5.6) is the fourth generation of IBM-PC software by the

same author,  and is the first to provide for operation with the PC  Jr.

This software was written by John Spalding of Atlanta Georgia and placed

by him in the public domain.

The  TDD was originally a mechanical teleprinter used with a unique

modem which employed the same tone pair (1400 Hz mark and 1800 Hz space)

in each direction.  The modem was developed by Dr.  Robert Weitbrecht, a

deaf physicist,  and was unfortunately incompatible  with  any  industry

standard modem. The TDD also uses the Baudot code rather than ASCII, and

so in order for a computer to talk to a TTY is has been  necessary  both

to  include  code  conversion  in the computer software and to provide a

separate modem.

Adaptive  software  has  been  developed  for  a  number   of  home

computers.  Details on this work are available from HEX.  Two computers,

the IBM-PC and the Radio Shack Color Computer, incorporate hardware that

permits  them to emulate a modem,  permitting their cassette ports to be

connected to the phone line via an inexpensive telephone amplifier. This

amplifier  may  be  built  by  the user,  or is available at cost (forty

dollars) from John E. Steele, 2100 N.  New Road, Waco, Texas 76707.  The

same amplifer is plug compatible with either the PC or the CoCo.

Similar software for the Apple,  Smoke Signal Broadcasting 6800 and

6809 machines,  Atari,  Pet,  TRS-80 Model I,  VIC-20  and  Commodore 64

computers can be obtained from the AMRAD, the sponsor of the HEX.  These

machines require addition of modems and  interface  circuitry.  Complete

details  can  be  obtained  by writing to the letterhead address,  or by

calling the HEX BBS at the number above.  Calls will be accepted  either

from an ASCII device (300 baud, 8 bits, no parity, one stop bit) or from

a standard TDD.

############################################################################